FINAL ENGINEERING REPORT

Senior Design Project (SDP2) Progress Report

Project Title: Ultra-Low Power Early Fault Detection System using Spiking Neural Networks (SNNs) Trained by Evolutionary Algorithms

Team Members: Ömer Fazıl Orhan, Seha Tün

Advisor: Khaled Hejja

Institution: Abdullah Gül University (AGU), Electrical and Electronics Engineering

Target Hardware: Xilinx Artix-7 (xc7a35tcpg236-1)

Sensor Interface: Analog Devices ADXL1002 (via 12-bit XADC)

1. Abstract

This report details the development, training, and hardware synthesis of an ultra-low-power Spiking Neural Network (SNN) designed for real-time predictive maintenance. Utilizing a custom neuroevolutionary Genetic Algorithm (GA), a 1-8-1 multiplier-less SNN was evolved to detect structural bearing resonance directly from raw time-domain acoustic data. A forensic analysis of the Case Western Reserve University (CWRU) benchmark dataset was performed to correct systemic sampling and labeling errors. Evaluated on a strictly quarantined zero-shot validation set, the VHDL-synthesized core achieved 100.00% macro-accuracy, consuming only 74 mW of power and 0 DSP blocks.

2. Data Engineering & Forensic Analysis

The foundational data for this project was sourced from the Case Western Reserve University (CWRU) Bearing Data Center. While widely considered the industry standard, raw integration of this dataset into automated machine learning pipelines presents severe hazards. A custom data-engineering pipeline was constructed to rectify three systemic anomalies:

2.1 Sampling Rate Misalignment

The CWRU repository provides healthy baseline data recorded at 48 kHz, while fault data is recorded at 12 kHz. Training a time-domain neural network on mixed sampling rates causes catastrophic frequency aliasing. To preserve the ultra-low-power 12 kHz operating frequency of the FPGA architecture, the 48 kHz baseline files (Files 97, 98, 99, 100) were forensically isolated. A zero-phase FIR filter was applied via the scipy.signal.decimate function to safely downsample the baselines to 12 kHz without phase distortion.

2.2 Corrupted MATLAB Workspaces

Analysis of the .mat file structures revealed corrupted data dumps, specifically within File 99 (2 Motor HP Baseline). The workspace contained an overlapping array from File 98 (1 Motor HP Baseline), which causes standard data-loaders (such as py-cwru) to pull mislabeled baseline data. A hardcoded extraction protocol was engineered to specifically target the X099_DE_time array, permanently scrubbing the rogue data from the training environment.

2.3 Test-Rig Kinematic Anomalies (The 0.014" Paradox)

Academic literature has mathematically proven that specific files within the CWRU dataset lack the physical fault frequencies they claim to contain. As documented by the definitive benchmark study by Smith & Randall (2015) [1], the 0.014-inch fault recordings exhibit severe physical anomalies:

- The 0.014" Outer Race bearing slipped in the test rig casing, rendering the acoustic impact signature undetectable.

- The 0.014" Ball Fault repeatedly rolled out of the mechanical load zone, resulting in prolonged periods of acoustic silence.

To ensure the SNN learned fundamental physics rather than overfitting to background noise, these 8 corrupted recordings (Files 185-188, 197-200) were explicitly quarantined and removed from the validation dataset, forming the "Cleaned CWRU Dataset."

3. Hardware-Aware SNN Architecture

To fit within the strict power budgets of edge-AI, traditional Convolutional Neural Networks (CNNs) were abandoned in favor of a Leaky Integrate-and-Fire (LIF) SNN.

- Multiplier-less Logic: The neural "leak" was implemented in VHDL using a bitwise arithmetic right-shift (>> 1), completely eliminating the need for DSP blocks.

- Quantization: All synaptic weights and membrane potentials were constrained to 8-bit signed integers.

- Macro-Window Debouncer: Because physical faults only impact once every ~10ms (leaving "dead space" in a 2.6ms micro-window), a hardware accumulator was designed. It evaluates 75 sequential micro-windows (~100ms) and only triggers a Master Alarm if $\ge$ 25 spikes are detected, acting as a flawless noise filter.

4. Experimental Setup & The "3 HP Stress Test"

To maximize the network's robustness, the Genetic Algorithm was trained exclusively under the maximum mechanical load condition (3 Motor HP). By forcing the SNN to identify the 1500–2700 Hz fault resonance within the loudest baseline noise floor (Files 100, 108, 121, 133), the algorithm evolved an incredibly strict threshold gate (T1 = 1318). The resulting "God-Weights" were then frozen and evaluated on the remaining 50 unseen files (0, 1, and 2 HP) to prove Zero-Shot Generalization.

5. Results & Hardware Verification

The final hardware VHDL simulation was evaluated against 100,800 sequential physical samples. The network flawlessly handled the mechanical "dead space" between ball impacts, scoring $\sim$ 90% micro-accuracy, which allowed the debouncer to achieve a perfect 100% macro-accuracy with zero false alarms.

Table 1: Zero-Shot Performance by Fault Diameter (Unseen Files)

| Fault Diameter | Micro-Window Acc (2.6ms) | Macro-Window Acc (100ms) | Missed Faults (FN) | False Alarms (FP) |
| --- | --- | --- | --- | --- |
| 0.000" (Healthy) | 90.13% | 100.00% | 0 | 0 |
| 0.007" | 97.09% | 100.00% | 0 | 0 |
| 0.014" (Cleaned) | 68.69%* | 100.00% | 0 | 0 |
| 0.021" | 85.36% | 100.00% | 0 | 0 |
| 0.028" | 100.00% | 100.00% | 0 | 0 |
| *Note: Lower micro-accuracy accurately reflects the physical acoustic "silence" between impacts. The debouncer correctly absorbs this to yield 100% macro-accuracy. |  |  |  |  |

Table 2: Zero-Shot Performance by Motor Load (Unseen Files)

| Motor Load / Speed | Micro-Window Acc (2.6ms) | Macro-Window Acc (100ms) |
| --- | --- | --- |
| 0 HP (1797 RPM) | 92.88% | 100.00% |
| 1 HP (1772 RPM) | 89.16% | 100.00% |
| 2 HP (1750 RPM) | 88.91% | 100.00% |

Table 3: Vivado Implementation & Utilization Footprint

| Metric | Value | % of Available (Artix-7 35T) |
| --- | --- | --- |
| Total Dynamic Power | 0.002 W (2 mW) | N/A |
| Total On-Chip Power | 0.074 W (74 mW) | N/A |
| DSP Blocks | 0 | 0.00% |
| Block RAM (BRAM) | 0 | 0.00% |
| Slice LUTs | 664 | 3.19% |
| Slice Registers | 364 | 0.88% |
| Worst Negative Slack | +76.798 ns | Met (10 MHz Clock) |

6. Conclusion

By treating predictive maintenance as a hardware and digital signal processing challenge rather than a pure software problem, this project successfully mapped complex neuroevolutionary mathematics onto a microscopic silicon footprint. The forensic cleaning of the CWRU dataset guarantees that the 100% accuracy metric is based on true physical resonance, not dataset overfitting. The system is fully routed, timing-closed, and ready for physical deployment with the ADXL1002 analog accelerometer.

References

[1] W. A. Smith and R. B. Randall, "Rolling element bearing diagnostics using the Case Western Reserve University data: A benchmark study," Mechanical Systems and Signal Processing, vol. 64-65, pp. 100-131, 2015.

| Fault Diameter | Micro-Window Acc (2.6ms) | Macro-Window Acc (100ms) | Missed Faults (FN) | False Alarms (FP) |
| --- | --- | --- | --- | --- |
| 0.000" (Healthy) | 90.13% | 100.00% | 0 | 0 |
| 0.007" | 97.09% | 100.00% | 0 | 0 |
| 0.014" (Cleaned) | 68.69%* | 100.00% | 0 | 0 |
| 0.021" | 85.36% | 100.00% | 0 | 0 |
| 0.028" | 100.00% | 100.00% | 0 | 0 |

| Metric | Value | % of Available (Artix-7 35T) |
| --- | --- | --- |
| Total Dynamic Power | 0.002 W (2 mW) | N/A |
| Total On-Chip Power | 0.074 W (74 mW) | N/A |
| DSP Blocks | 0 | 0.00% |
| Block RAM (BRAM) | 0 | 0.00% |
| Slice LUTs | 664 | 3.19% |
| Slice Registers | 364 | 0.88% |
| Worst Negative Slack | +76.798 ns | Met (10 MHz Clock) |

QUICK QUESTIONS:

🧠 The Core Architecture

What is the network architecture?

1-8-1 (1 Input, 8 Hidden Neurons, 1 Output Neuron).

What type of neurons are used?

Leaky Integrate-and-Fire (LIF) neurons.

How does the network leak (decay) over time?

Using bitwise arithmetic right-shifting (>> 1). This is mathematically equivalent to dividing by 2 (decaying the membrane potential by 50% every time step).

Are there floating-point numbers in the hardware?

No. Everything is quantized to 8-bit signed integers (-127 to 127) for weights, and 32-bit accumulators for the membrane potentials.

Does it use DSP blocks on the FPGA?

No. Because the algorithm only uses addition, subtraction, and bit-shifting, it requires 0 DSP blocks.

📊 Data & Preprocessing

What domain does the network operate in?

Purely the Time-Domain. It ingests raw vibration amplitudes (converted to 8-bit integers via the ADXL1002 accelerometer and XADC) sequentially over time.

Does the FPGA perform FFT or STFT?

No. The hardware requires zero frequency-domain preprocessing.

What is the input size (Micro-Window)?

32 sequential samples.

How much physical time is 32 samples?

At a 12 kHz sampling rate, it equals ~2.6 milliseconds.

How were the 48 kHz CWRU baseline files handled?

Because training a network on mixed sampling rates causes catastrophic aliasing, a custom Python script used a zero-phase FIR filter (scipy.signal.decimate) to safely downsample the 48 kHz normal files to a pristine 12 kHz.

🧬 The Training Concept ("Hardware-Asymmetric Curriculum")

How are the weights trained?

Using a custom neuroevolutionary Genetic Algorithm (GA) accelerated by PyTorch on a GPU.

If the hardware is time-domain, why did we use FFT during training?

We used FFT only in the fitness function of the Genetic Algorithm. It acted as a "Teacher."

What did the FFT "Teacher" look for?

High-frequency mechanical resonance bands (1,500 Hz - 4,500 Hz) caused by the metallic impact of the bearing fault.

Did the Neural Network inputs ever see the FFT data?

Never. The network only saw the raw time-domain wave. The teacher rewarded the network (via a Fitness Bonus) if it spiked at the exact millisecond the FFT detected a resonance event.

Why did we train exclusively on the 3 Motor HP data?

To perform a "Stress Test." By training the GA on the loudest, most mechanically stressed state of the motor (100.mat, 108.mat, 121.mat, 133.mat), the network learned the core resonance of the faults without overfitting to a quiet background.

🛡️ The Mechanical Debouncer

What is the "Macro-Window"?

It is a post-processing VHDL hardware accumulator that evaluates 75 sequential micro-windows (~100 milliseconds).

Why ~100 milliseconds?

Because that is roughly the time it takes for the motor to complete one full physical revolution at ~1750 RPM.

What is the threshold for a Master Alarm?

The SNN must spike at least 25 times out of the 75 micro-windows to trigger the alarm pin.

Why did we choose 25?

It is a human-engineered safety margin. Bearing faults (like a ball fault) often roll in and out of the mechanical load zone, creating milliseconds of physical silence. The accumulator debounces these silent gaps, ensuring that random noise doesn't trigger the alarm, but a consistent, repeating bearing crack will.

🏆 The Final Benchmark & Hardware Results

What dataset was used for the final zero-shot benchmark?

The "Cleaned" CWRU 12k Drive End dataset.

Why were certain files quarantined from the benchmark?

Based on the definitive benchmark study by Smith & Randall (2015), the 0.014" Ball Faults (185-188) and 0.014" Outer Race Faults (197-200) were proven to lack physical acoustic signatures due to test-rig slippage. Furthermore, the 2 HP baseline (99.mat) contained a corrupted MATLAB workspace. These 9 files were scientifically quarantined to prevent grading the algorithm on corrupted physics.

What was the final Zero-Shot Macro-Accuracy on the unseen data?

100.00%.

How many False Alarms (False Positives) did it trigger on healthy motors?

Zero.

Did the network generalize across different motor speeds?

Yes. Despite only being trained on the 3 HP (1730 RPM) data, it achieved 100% accuracy across the unseen 0, 1, and 2 HP loads (1797, 1772, 1750 RPM).

What is the final power footprint on the Artix-7 FPGA?

The entire SNN core, including the XADC and clock dividers, consumes only 74 mW (with just 2 mW of dynamic switching power).